Leica LAS X metadata file from DMI8 microscope

Image Dimensions & Resolution

- Image size: 1200 × 1200 pixels

- Physical size: 131.39 μm × 131.39 μm

- Pixel size: ~109.5 nm/pixel (calculated from physical size)

- Bit depth: 12-bit (0-4095 intensity range)

- Number of channels: 2 (Grayscale and Red)

Optical Configuration

- Objective: N PLAN 100x/1.25 Oil (Art. No. 11506158)

- Numerical Aperture (NA): 1.25

- Immersion: Oil (refractive index 1.518)

- Microscope: DMI8 (inverted)

- Total magnification: 100x

Channel Information

Channel 1 (Brightfield - "New 1")

- Contrast method: TL-BF (Transmitted Light Brightfield)

- Exposure: 138 ms

- Transmitted light intensity: 112

- LUT: Gray

- Filter cube: EMP_BF

Channel 2 (Fluorescence - "New 2")

- Contrast method: FLUO

- Exposure: 138 ms

- Filter cube: DFT51010

- Emission wavelength: 594 nm

- LUT: Red

- LED wavelength 555 nm at 84% intensity (active)

- Field/Aperture diaphragm settings available

Stage & Focus Information

- Stage position: X = 61.62 mm, Y = 39.78 mm

- Z position: 2.539 mm

- Z-mode: z-wide

- Autofocus: Combined HSAF system, channel 1, precision level 2

Camera Settings

- Camera: Photometric Prime 95B (model A21F203002)

- Gain: Full well mode

- Temperature: -18°C (target: -30°C)

- Fan control: Active (level 3)

- ROI: Full frame (no cropping)

Display/Viewing Settings

- Channel 1 scaling: Black = 4.4%, White = 18.8%

- Channel 2 scaling: Black = 4.4%, White = 7.3%